# §250113_Experimental-Plan-Drying

Metadata

Topic:: Experimental plan to measure drying on conductive electrodes
Context:: SFB1615 A07
Devices:: {Zurich Impedance Analyzer,Profilometer}

# Target

- Validate the impedance curve of conductive PCBs during drying. Have a bigger initial water droplet or more measurement points during the drying process to track all phases and validate previous measurements.

- The goal is to have see all phases of the drying process

- Try avoiding lift-off effect

# Materials

- Zurich Impedance Analyzer

  - -

  - Impedance range = auto

  - Voltage and current range = auto

- Profiometer

  - Focus on the electrode position not the whole layer

- PCBs

  - Conductive

  - Printed in Freiburg

- Others

  - Pipette

  - sodium benzoate solution

# Procedure

- Measure the weight of the PCB

- Droplets of  sodium benzoate solution on top of the pcb electrodes

  - Spread the solution to form a big layer

- Measure weight after applying the droplet

- Drying in oven at  until it is dry

  - Measure during drying:

    - Oven intervals of 2min

    - Take PCB out of oven -> measure impedance -> measure weight

- Stop the measurements, after the 3rd sample with  phase angle in the impedance

- Repeat this measurement at least 2 times to achieve a versatile imagine like the one below